Project 1: The Mystery Binary

Himadri Saha

EECE.5821 Computer Architecture

10/7/2026

Part C

C.1

This program reads the integer n and prints the sum of the squares.

n = read_int()

for i = 1..n:  arr[i-1] = mult(i, i)      # arr at 0x10010840

sum = arr[0] + ... + arr[n-1]

print "=> ", sum, '\n'  then exit

mult(a, b) multiplies by repeated addition (b loops)

C.2

ic_n10 = 441

C.3

IC(n) = Σ sᵢ · kᵢ(n):

IC(n) = 6(1) + 1(n+1) + 3n + 5n + 2(1) + 1(n+1) + 6n + 11(1) + 1n + 1[n(n+1)/2 + n] + 3[n(n+1)/2] + 2n

(B9 and B10):

(1 + 3) · n(n+1)/2 = 4 · n(n+1)/2 = 2n(n+1) = 2n² + 2n

remaining n terms:

n (B1) + 3n (B2) + 5n (B3) + n (B5) + 6n (B6) + n (B8) + n (B9) + 2n (B11) = 20n

Collect the constants:

6 (B0) + 1 (B1) + 2 (B4) + 1 (B5) + 11 (B7) = 21

Solve:

IC(n) = (2n² + 2n) + 20n + 21

IC(n) = 2n² + 22n + 21

a = 2, b = 22, c = 21

C.6

- ALU contributes to the most cycles, it’s the most common class and has the highest CPI amongst the common classes (4 compared with 3 for branches and jumps)

- f = 72 100 / 78 206 = 0.922 ->92.2%

- Speedup = 1 / [(1 − f) + f/s]

As s → ∞, the f/s term goes to 0:

Speedup_max = 1 / (1 − f) = total cycles / (cycles not in mult)

Numbers:

Speedup_max = 78 206 / (78 206 − 72 100) = 78 206 / 6 106 = 12.81×

Part D

| # | Optimization | IC Changes | CPI Changes | Why it helps |
| --- | --- | --- | --- | --- |
| 1 | Closed-form sum: Σi² = n(n+1)(2n+1)/6 replaces both loops | Large drop: O(n²) → constant | Rises (uses 30-cycle mul/div) | Removes all the repeated work. This is an algorithm change, not a small tweak. |
| 2 | No array: squares are never stored and reloaded | Drops by 2n (n sw + n lw) | Removes 4-CPI stores and 5-CPI loads | The array was only temporary storage, and memory operations are the most expensive non-MulDiv class. |
| 3 | No subroutine call: mult is gone entirely | Drops by 4n (jal, jalr, setup, return move) | — | Removes call overhead along with the loop inside it. |
| 4 | Strength reduction: ÷2 done with srli instead of div | — (still 1 instruction) | Drops: 4 cycles instead of 30 | n(n+1) is always even, so a shift gives the exact result. Saves 26 cycles. |
| 5 | 2n + 1 computed as add t2, a0, t0, i.e. n + (n+1) | Drops by 1 (instead of slli + addi) | — | Reuses the n+1 already computed. Saves 4 cycles. |
| 6 | Removed the redundant addi a0, a0, 0 after lui | Drops by 1 | — | The string is at 0x10010000, so its low 12 bits are 0 and lui alone builds the full address. Saves 4 cycles. |
| 7 | Reordered to compute the result straight into a0 | Drops by 1 (no mv before print-int) | — | The work starts while a0 still holds n, so the final divu writes the value to print directly into a0. Saves 4 cycles. |
| 8 | Negative-input guard (bge) | Rises by 1 | — | Costs 3 cycles. Kept for correctness: the original prints 0 for n < 1, but the formula alone would print nonzero values for some negative n (for example, n = −2 gives −1). |

RV32M is worth it for any meaningful n. Each 30-cycle instruction replaces work that grows with n, so its fixed cost pays for itself quickly. For very small inputs, though, a pure RV32I version would be faster.

When mul costs 30 cycles:

i	Repeated addition (14i + 14)	mul (30)	Winner

1	28	30	repeated addition

2	42	30	mul

10	154	30	mul (5×)

100	1 414	30	mul (47×)

Without RV32M, the best approach is an O(n) loop that builds each square from the previous one with no multiply:

(i+1)² = i² + (2i + 1)

Total ≈ 85 + 19n cycles.

For which n is the optimized version not much faster?

| n | Original cycles (7n² + 81n + 106) | Optimized | Speedup |
| --- | --- | --- | --- |
| 0 | 106 | 177 | 0.60× (slower) |
| 1 | 194 | 177 | 1.10× |
| 2 | 296 | 177 | 1.67× |
| 5 | 686 | 177 | 3.88× |
| 10 | 1 616 | 177 | 9.13× |
| 20 | 4 526 | 177 | 25.6× |
| 100 | 78 206 | 177 | 441.8× |

At n <= 2, optimized version is faster. At n = 0, the version is slower. This is because the original version cost grows with n, but the optimized cost is fixed. At smaller values for n, the original n2 is ~0, meaning it costs a little more than setup and printing instructions.

Graduate component

G1.